# Computer Networking: Concepts Experiment 6

# PART 1 — FIXED-LENGTH SUBNETTING

Given Network: 172.16.0.0/24
Required Subnets:

- Subnet A: 40 hosts

- Subnet B: 20 hosts

- Subnet C: 10 hosts

- Subnet D: 5 hosts

### ❗ Fixed-length subnetting means all subnets must be of equal size.

# Step 1 — Determine Subnet Size

We need the largest host requirement → 40 hosts.

Formula:

Hosts = 2^h – 2  ≥ 40

2^6 – 2 = 62  → OK

So we need 6 host bits, leaving:

Prefix = 32 – 6 = /26

Subnet mask = 255.255.255.192

Each subnet size = 2⁶ = 64 addresses per subnet

# Step 2 — Create Subnets of /26

| Subnet | Network Address | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- |
| A | 172.16.0.0/26 | 172.16.0.1 | 172.16.0.62 | 172.16.0.63 |
| B | 172.16.0.64/26 | 172.16.0.65 | 172.16.0.126 | 172.16.0.127 |
| C | 172.16.0.128/26 | 172.16.0.129 | 172.16.0.190 | 172.16.0.191 |
| D | 172.16.0.192/26 | 172.16.0.193 | 172.16.0.254 | 172.16.0.255 |

# Step 3 — Packet Tracer Topology

Use:

- 1 Router

- 4 Switches

- Multiple PCs in each subnet

To add new interfaces (Extra Ports)

### Drag the Router to the Workspace

- From Devices → Routers

- Choose Cisco 2911

- Drag onto the workspace.

### Power OFF the Router

You cannot insert modules while powered on.

- Click the router

- Go to the Physical tab

- Find the Power button (left side → switch)

- Click it → It turns red (OFF)

### Choose a Module to Add

Common modules for extra ports:

| Module Name | Ports Added | Purpose |
| --- | --- | --- |
| HWIC-2T | 2 Serial ports | WAN/Serial links |
| HWIC-4ESW | 4 FastEthernet switch ports | LAN expansion |
| HWIC-1GE-SFP | 1 GigE | High-speed port |
| NIM-ES2-4 | 4 Ethernet ports | Newer PT versions |

For subnetting labs, the HWIC-4ESW is the easiest and most used.

### Insert the Module

- In the router’s Physical tab, you’ll see empty HWIC slots.

- Drag the module (e.g., HWIC-4ESW) from the left panel

- Drop it into an empty slot.

You will see the module appear with new ports like:

Fa0/0

Fa0/1

Fa0/2

Fa0/3

### Power ON the Router

- Click the same power switch

- It turns green (ON)

### Go to CLI and Verify Ports

Open CLI → type:

show ip interface brief

You should now see the new interfaces.

Connect:

# Step 4 — Router Configuration

Example for Router:

enable

configure terminal

interface g0/0

ip address 172.16.0.1 255.255.255.192

no shutdown

interface g0/1

ip address 172.16.0.65 255.255.255.192

no shutdown

interface g0/2

ip address 172.16.0.129 255.255.255.192

no shutdown

interface g0/3

ip address 172.16.0.193 255.255.255.192

no shutdown

Since a single router is used, routing is automatic (no static routes needed).

# Step 5 — PC Configuration Example

PC in Subnet A:

IP: 172.16.0.10

Mask: 255.255.255.192

Gateway: 172.16.0.1

Repeat for all PCs.

# Step 6 — Verification

Ping across subnets:

PC-A → PC-B

PC-A → PC-C

PC-A → PC-D

All pings should succeed.

# PART 2 — VLSM

# We use the same network 172.16.0.0/24, but assign subnet sizes based on host counts.

Host requirements:

- A → 40

- B → 20

- C → 10

- D → 5

# Step 1 — Sort by host size

- A = 40 hosts → needs /26 (62 hosts)

- B = 20 hosts → /27 (30 hosts)

- C = 10 hosts → /28 (14 hosts)

- D = 5 hosts → /29 (6 hosts)

# Step 2 — Allocate Subnets (largest first)

### ✔ Subnet A (40 hosts) → /26

172.16.0.0/26

Range: 172.16.0.1 – 172.16.0.62

Broadcast: 172.16.0.63

### ✔ Subnet B (20 hosts) → /27

Next available block starts at 172.16.0.64

172.16.0.64/27

Range: 172.16.0.65 – 172.16.0.94

Broadcast: 172.16.0.95

### ✔ Subnet C (10 hosts) → /28

Next block starts at 172.16.0.96

172.16.0.96/28

Range: 172.16.0.97 – 172.16.0.110

Broadcast: 172.16.0.111

### ✔ Subnet D (5 hosts) → /29

Next block starts at 172.16.0.112

172.16.0.112/29

Range: 172.16.0.113 – 172.16.0.118

Broadcast: 172.16.0.119

# Step 3 — Router Interface Assignment

Do same as previous

# Step 4 — Configure PCs

Example for Subnet C:

IP: 172.16.0.100

Mask: 255.255.255.240

Gateway: 172.16.0.97

# Step 5 — Verify With Ping

All hosts must successfully ping across all networks.